# Programming for Robotics – Project Pitch

## Project Title:

Smart Hazard Detection System

## Group Members:

- Ayush Patel (s5353051)

- Mohammad Rashed Khan (s5404089)

- Lachlan Roche (s542960)

## Target Specialisation:

Option A – Embedded Intelligence (Wokwi)

## 1. Executive Summary

The Smart Hazard Detection System is an embedded intelligent system designed to monitor environmental conditions and detect hazardous situations in real time. It is suitable for high-risk environments such as industrial facilities and fire-prone zones where early detection is critical.

The system integrates multiple sensors and applies sensor fusion with multi-condition decision logic to classify conditions into safe, warning, and emergency states. Based on these conditions, the system responds through visual, auditory, and physical outputs, ensuring a responsive and context-aware monitoring solution.

## 2. System Architecture (Sense–Think–Act)

### Sensing (Inputs):

- DHT22 Sensor: Measures temperature and humidity

- Ultrasonic Sensor: Detects nearby objects and environmental presence

### Processing (Thinking):

- Finite State Machine (FSM): Controls system behaviour using structured states

- Decision Logic: Uses multi-condition evaluation combining temperature and humidity (sensor fusion)

### Actuation (Outputs):

- Servo Motor: Performs scanning (0°–180°)

- LED Indicator: Green (Safe), Yellow (Warning), Red (Emergency)

- Buzzer: Provides alert signals during hazard conditions

## 3. Behavioural Logic (Finite State Machine)

The system implements a 5-state FSM to ensure structured and adaptive behaviour:

| State | Trigger Condition | Behaviour |
| --- | --- | --- |
| IDLE | Safe conditions | Green LED, low monitoring |
| SURVEY | Periodic timer | Servo scanning, active sensing |
| CAUTION | Temp > 45°C OR Humidity < 20% | Yellow LED, intermittent buzzer |
| EMERGENCY | Temp > 60°C AND Humidity < 15% | Red LED blinking, continuous buzzer |
| FAILSAFE | Sensor error / invalid data | Safe mode, restricted outputs |

This structure ensures multi-condition logic, clear transitions, and stable operation.

## 4. Specialisation: Embedded Intelligence

The system demonstrates embedded intelligence through:

- Sensor Fusion: Uses combined temperature and humidity for hazard detection

- Context-Aware Behaviour: Adjusts system response based on severity

- Real-Time Logic: Continuously monitors and reacts without delay-based blocking

## 5. Safety, Robustness & Reliability

The system includes:

- Fail-safe mechanism: Handles sensor errors and invalid readings

- Controlled FSM behaviour: Prevents unpredictable system states

- Emergency alerts: Immediate visual and auditory response

## 6. Advanced Component (10%)

The system includes adaptive behaviour by:

- Adjusting alert intensity based on hazard level

- Increasing monitoring frequency during critical conditions

- Implementing intelligent scanning using servo motion